OnPrem Multi-Vendor
EVPN VXLAN Integration

Cisco Catalyst 9300  •  Versa FlexVNF  •  Cumulus Linux (NVIDIA)

Version 3.0  |  February 16, 2026  |  INTERNAL

## 1. Summary

This document covers the integration of a multi-vendor BGP EVPN VXLAN fabric for Layer 2 extension of VLAN 1151 (VNI 401151) across four leaf VTEPs. The fabric uses iBGP with a route reflector, OSPF underlay, and ingress replication for BUM traffic.

| Component | Status |
| --- | --- |
| OSPF Adjacencies | FULL (all 4 peers) |
| BGP EVPN Peers | Established (4/4) |
| VXLAN Tunnels | 4 VTEPs UP |
| Cross-vendor MAC Learning | Working |
| VXLAN-GBP (Multi-Vendor) | FIXED |

## 2. Topology

All four leaf VTEPs connected to sw1 spine. VXLAN full-mesh tunnels overlay. Cisco (with SGT) and Versa send VXLAN-GBP headers.

## 3. Device Inventory

| Device | Role | Platform | Loopback/VTEP | OSPF RID | BGP AS | Mgmt IP |
| --- | --- | --- | --- | --- | --- | --- |
| [CISCO] sw1 | Spine / BGP RR | Catalyst 9300 | 1.1.1.1 | 1.1.1.1 | 6500 | 192.168.20.77 |
| [CISCO] sw2 | Leaf VTEP | Catalyst 9300 | 1.1.1.2 | 1.1.1.2 | 6500 | 192.168.20.76 |
| [CISCO] sw3 | Leaf VTEP | Catalyst 9300 | 1.1.1.3 | 1.1.1.3 | 6500 | 192.168.20.81 |
| [VERSA] FlexVNF | SD-WAN VTEP | FlexVNF 22.1.4 | 1.1.1.4 | 12.12.12.10 | 6500 | 192.168.20.166 |
| [CUMULUS] Nvidia | Leaf VTEP | Cumulus Linux 5.x | 1.1.1.5 | 1.1.1.5 | 6500 | 192.168.20.172 |

### Underlay Point-to-Point Links

| Link | sw1 Intf | sw1 IP | Remote Intf | Remote IP | Subnet |
| --- | --- | --- | --- | --- | --- |
| sw1 <-> sw2 | Gi1/0/1 | 12.12.12.1 | Gi1/0/1 | 12.12.12.2 | 12.12.12.0/30 |
| sw1 <-> sw3 | Gi1/0/2 | 12.12.12.5 | Gi1/0/1 | 12.12.12.6 | 12.12.12.4/30 |
| sw1 <-> Versa | Gi1/0/3 | 12.12.12.9 | Vni-0/2 | 12.12.12.10 | 12.12.12.8/30 |
| sw1 <-> Cumulus | Gi1/0/4 | 12.12.12.13 | swp1 | 12.12.12.14 | 12.12.12.12/30 |

## 4. VLAN / VNI Mapping

| VLAN | Name | VNI | Subnet | Purpose | Devices |
| --- | --- | --- | --- | --- | --- |
| 1101 | VRF1_CORE_VLAN | 50101 | — | L3VNI (Core Routing) | sw2, sw3 |
| 1151 | VRF1_ACCESS_VLAN | 401151 | 192.168.1.0/25 | L2 Extension (All VTEPs) | sw2, sw3, Versa, Cumulus |
| 1152 | VRF1_ACCESS_VLAN_2 | 401152 | 192.168.2.0/25 | Access VLAN | sw2, sw3 |
| 1153 | VRF1_ACCESS_VLAN_3 | 401153 | 192.168.3.0/25 | Access VLAN | sw2, sw3 |

## 5. Underlay Configuration (OSPF)

### 5.1 sw1 — Spine (Cisco IOS-XE)

! sw1 - OSPF underlay + interfaces

router ospf 100

router-id 1.1.1.1

auto-cost reference-bandwidth 100000

interface Loopback0

ip address 1.1.1.1 255.255.255.255

ip ospf 100 area 100

interface GigabitEthernet1/0/1

description to sw2

no switchport

ip address 12.12.12.1 255.255.255.252

ip ospf network point-to-point

ip ospf 100 area 100

interface GigabitEthernet1/0/2

description to sw3

no switchport

ip address 12.12.12.5 255.255.255.252

ip ospf network point-to-point

ip ospf 100 area 100

interface GigabitEthernet1/0/3

description to Versa FlexVNF

no switchport

ip address 12.12.12.9 255.255.255.252

ip mtu 1550

ip ospf network point-to-point

ip ospf mtu-ignore

ip ospf 100 area 100

interface GigabitEthernet1/0/4

description to Cumulus_Nvidia

no switchport

ip address 12.12.12.13 255.255.255.252

ip mtu 1550

ip ospf network point-to-point

ip ospf mtu-ignore

ip ospf 100 area 100

Warning: MTU Mismatch: Cumulus defaults to MTU 9216. Cisco uses 1550. ip ospf mtu-ignore is configured on both ends to prevent OSPF ExStart/DBD failures.

### 5.2 Versa — Underlay OSPF

routing-instances routing-instance Underlay {

instance-type virtual-router

networks      [​ OSPF_Underlay ]

evpn-core

interfaces    [​ tvi-0/9005.0 ]

policy-options {

redistribution-policy Export-Vtep-To-OSPF {

term VTEP_IP {

match { address 1.1.1.4/32 }

action { accept }

}

}

redistribute-to-ospf Export-Vtep-To-OSPF

}

protocols {

ospf 4023 {

router-id     12.12.12.10

area 100 {

network-name OSPF_Underlay {

network-type    point-to-point

hello-interval  10

dead-interval   40

}

}

}

}

}

### 5.3 OSPF Verification (sw1)

sw1# show ip ospf neighbor

Neighbor ID     Pri   State           Dead Time   Address         Interface

12.12.12.10       0   FULL/  -        00:00:32    12.12.12.10     GigabitEthernet1/0/3

1.1.1.3           0   FULL/  -        00:00:39    12.12.12.6      GigabitEthernet1/0/2

1.1.1.2           0   FULL/  -        00:00:31    12.12.12.2      GigabitEthernet1/0/1

1.1.1.5           0   FULL/  -        00:00:37    12.12.12.14     GigabitEthernet1/0/4

Status: OSPF Status: All four adjacencies FULL — sw2, sw3, Versa, Cumulus.

## 6. BGP EVPN Overlay

### 6.1 sw1 — Route Reflector

router bgp 6500

bgp router-id interface Loopback0

bgp log-neighbor-changes

template peer-session spine-peer-session

remote-as 6500

update-source Loopback0

exit-peer-session

neighbor 1.1.1.2 inherit peer-session spine-peer-session

neighbor 1.1.1.3 inherit peer-session spine-peer-session

neighbor 1.1.1.4 inherit peer-session spine-peer-session

neighbor 1.1.1.4 send-community both

neighbor 1.1.1.5 inherit peer-session spine-peer-session

neighbor 1.1.1.5 send-community both

address-family l2vpn evpn

neighbor 1.1.1.2 activate

neighbor 1.1.1.2 send-community both

neighbor 1.1.1.2 route-reflector-client

neighbor 1.1.1.3 activate

neighbor 1.1.1.3 send-community both

neighbor 1.1.1.4 activate

neighbor 1.1.1.4 send-community both

neighbor 1.1.1.4 route-reflector-client

neighbor 1.1.1.5 activate

neighbor 1.1.1.5 send-community both

neighbor 1.1.1.5 route-reflector-client

exit-address-family

### 6.2 Versa — BGP EVPN

routing-instances routing-instance Underlay {

protocols {

bgp 3023 {

router-id     1.1.1.4

local-as { as-number 64515 }

group vxlan_internal {

type     internal

family { l2vpn { evpn } }

local-as 6500

neighbor 1.1.1.1 {

local-address tvi-0/9005.0

}

}

}

}

}

Information: Note: Versa uses local-as 6500 override to match the fabric AS while keeping its internal AS 64515.

### 6.3 BGP Verification (sw1)

sw1# show bgp l2vpn evpn summary

BGP router identifier 1.1.1.1, local AS number 6500

Neighbor        V    AS   MsgRcvd MsgSent  TblVer InQ OutQ Up/Down  State/PfxRcd

1.1.1.2         4  6500      224     290     106    0    0 03:03:30       18

1.1.1.3         4  6500      234     233     106    0    0 03:04:07       15

1.1.1.4         4  6500      429     454     106    0    0 03:03:42        1

1.1.1.5         4  6500     1148    1130       0    0    0 00:48:57        2

Status: BGP Status: All four EVPN peers established. Cumulus: 2 routes (Type-2 + Type-3). Versa: 1 route (Type-3 IMET).

## 7. L2VPN EVPN Instances

### 7.1 Route Target Mapping

| Device | RD | Export RT | Import RT |
| --- | --- | --- | --- |
| [CISCO] sw2 | 1.1.1.2:1151 (auto) | 1151:1 | 1151:1, 65000:1151, 2:2 |
| [CISCO] sw3 | 1.1.1.3:1151 (auto) | 1151:1 | 1151:1 |
| [VERSA] | 2L:122 | 2:2, 1151:1 | 2:2, 1151:1 |
| [CUMULUS] | 1.1.1.5:2 (auto) | 1151:1 | 1151:1 |

Warning: Route Target: Versa exports RT 2:2 in addition to 1151:1. All devices import 1151:1 so Versa routes are received.

### 7.2 sw2 L2VPN & NVE

l2vpn evpn instance 1151 vlan-based

encapsulation vxlan

route-target export 1151:1

route-target import 1151:1

route-target import 65000:1151

route-target import 2:2

!

vlan configuration 1151

member evpn-instance 1151 vni 401151

!

interface nve1

no ip address

source-interface Loopback0

host-reachability protocol bgp

member vni 50101 vrf VRF-1

member vni 401151 ingress-replication

member vni 401152 ingress-replication local-routing

member vni 401153 ingress-replication local-routing

### 7.3 Versa Virtual-Switch

routing-instances routing-instance VERSA-default-switch {

instance-type       virtual-switch

bridge-options { l2vpn-service-type vlan }

bridge-domains {

VLAN-1151 {

vlan-id 1151

bridge-options { bridge-domain-arp-suppression enable }

vxlan { vni 401151 }

}

}

interfaces          [​ vni-0/1.1 ]

route-distinguisher 2L:122

vrf-both-target     "target:2L:2 target:1151:1"

protocols {

evpn {

vlan-id-list  [​ 1151 ]

encapsulation vxlan

core-instance Underlay

}

}

}

## 8. Cumulus Linux (NVIDIA) — NVUE Configuration

Information: Platform: Cumulus Linux 5.x uses NVUE (nv set) as the primary configuration method. Apply changes with nv config apply -y.

### 8.1 Complete NVUE Commands

# -- Loopback (VTEP Source) --

nv set interface lo type loopback

nv set interface lo ip address 1.1.1.5/32

# -- Underlay Link to sw1 --

nv set interface swp1 type swp

nv set interface swp1 link state up

nv set interface swp1 ip address 12.12.12.14/30

nv set interface swp1 link mtu 9216

# -- OSPF Underlay --

nv set router ospf enable on

nv set router ospf router-id 1.1.1.5

nv set vrf default router ospf enable on

nv set vrf default router ospf router-id 1.1.1.5

nv set interface lo router ospf enable on

nv set interface lo router ospf area 100

nv set interface swp1 router ospf enable on

nv set interface swp1 router ospf area 100

nv set interface swp1 router ospf network-type point-to-point

nv set interface swp1 router ospf mtu-ignore on

# -- BGP EVPN Overlay --

nv set router bgp enable on

nv set router bgp autonomous-system 6500

nv set router bgp router-id 1.1.1.5

nv set vrf default router bgp enable on

nv set vrf default router bgp neighbor 1.1.1.1 remote-as internal

nv set vrf default router bgp neighbor 1.1.1.1 type numbered

nv set vrf default router bgp neighbor 1.1.1.1 update-source lo

nv set vrf default router bgp address-family l2vpn-evpn enable on

nv set vrf default router bgp neighbor 1.1.1.1 address-family l2vpn-evpn enable on

# -- VXLAN / NVE --

nv set nve vxlan enable on

nv set nve vxlan source address 1.1.1.5

nv set evpn enable on

# -- Bridge + VLAN 1151 + VNI --

nv set bridge domain br_default vlan 1151 vni 401151

nv set evpn vni 401151 route-target export 1151:1

nv set evpn vni 401151 route-target import 1151:1

# -- Trunk Port (swp2) - VLAN 1 native + VLAN 1151 tagged --

nv set interface swp2 type swp

nv set interface swp2 link state up

nv set interface swp2 bridge domain br_default

nv set interface swp2 bridge domain br_default stp admin-edge on

nv set interface swp2 bridge domain br_default stp bpdu-guard on

# NOTE: swp2 defaults to trunk mode (VLAN 1 PVID + all bridge VLANs tagged)

# Client uses tagged VLAN 1151 subinterface. Do NOT set access mode.

# -- SVI for VLAN 1151 --

nv set interface vlan1151 type svi

nv set interface vlan1151 vlan 1151

nv set interface vlan1151 ip address 192.168.1.15/25

# -- Apply --

nv config apply -y

### 8.2 Cumulus Verification

cumulus@cumulus:~$ sudo vtysh -c "show ip ospf neighbor"

Neighbor ID     Pri State           Dead Time Address         Interface

1.1.1.1           1 Full/DROther      39.892s 12.12.12.13     swp1:12.12.12.14

cumulus@cumulus:~$ sudo vtysh -c "show bgp l2vpn evpn summary"

BGP router identifier 1.1.1.5, local AS number 6500 vrf-id 0

Peers 1, using 23 KiB of memory

Neighbor        V   AS   MsgRcvd  MsgSent  Up/Down  State/PfxRcd  PfxSnt

1.1.1.1         4  6500    1148     1130   00:48:57           25       2

cumulus@cumulus:~$ sudo vtysh -c "show evpn vni 401151"

VNI: 401151

Type: L2

Vlan: 1151

Bridge: br_default

VxLAN interface: vxlan48

Local VTEP IP: 1.1.1.5

Remote VTEPs for this VNI:

1.1.1.4 flood: HER

1.1.1.3 flood: HER

1.1.1.2 flood: HER

Number of MACs (local and remote): 6

Number of ARPs (local and remote): 6

Status: Cumulus EVPN Status: VNI 401151 active. 3 remote VTEPs discovered. All remote MACs learned via EVPN Type-2 routes.

## 9. NVE Peer Verification

sw2# show nve peers vni 401151

Interface  VNI      Type Peer-IP     RMAC/Num_RTs  eVNI     state  UP time

nve1       401151   L2CP 1.1.1.3     3             401151     UP   06:01:13

nve1       401151   L2CP 1.1.1.4     1             401151     UP   00:49:17

nve1       401151   L2CP 1.1.1.5     2             401151     UP   00:48:57

cumulus@cumulus:~$ ip -d link show vxlan48

14: vxlan48:  mtu 9216

master br_default state UNKNOWN

vxlan id 401151 local 1.1.1.5 dstport 4789

nolearning ttl 64 udpcsum gbp

bridge_slave state forwarding learning off neigh_suppress on

Information: Key Parameters: nolearning = EVPN controls FDB, neigh_suppress = ARP suppression, gbp = VXLAN-GBP enabled (required for interop with any GBP-sending VTEP: Cisco with SGT/GBP, Versa, etc.).

## 10. MAC Address Learning

### Versa Bridge Table

admin@Cisco-EVPN-Br1-cli> show bridge

ROUTING INSTANCE         BRIDGE     MAC-ADDRESS        TYPE  VLAN  TUNNEL EP

VERSA-default-switch     VLAN-1151  aa:bb:cc:00:27:00  CA    1151  1.1.1.3

VERSA-default-switch     VLAN-1151  aa:bb:cc:00:24:00  CA    1151  1.1.1.2

### sw2 EVPN MAC Table

sw2# show l2vpn evpn mac evi 1151

MAC Address    EVI   VLAN  Next Hop(s)

aabb.cc00.2400 1151  1151  Gi1/0/4:1151    (local)

aabb.cc00.2700 1151  1151  1.1.1.3         (remote - sw3)

aabb.cc00.2f10 1151  1151  1.1.1.4         (remote - Versa)

Status: Cross-Vendor MAC Learning Confirmed: All VTEPs see each other's client MACs via EVPN Type-2 routes.

## 11. VXLAN-GBP Interoperability Issue

Issue: NET-2026-001 — Severity: HIGH: 100% data plane packet loss from any GBP-sending VTEP to Cumulus Linux. EVPN control plane is fully operational (BGP sessions up, routes exchanged, NVE peers visible). Root cause: Multiple vendors send VXLAN packets with GBP flags (0x88) — including Cisco Catalyst 9300 with group-based-policy + CTS/SGT enabled, and Versa FlexVNF (GBP always on by default). The Linux kernel VXLAN driver silently drops GBP-flagged packets on non-GBP VXLAN interfaces. This is not a vendor-specific issue — it is a Linux kernel behavior.

### 11.1 What is VXLAN-GBP?

VXLAN-GBP (Group Based Policy) is an extension to the standard VXLAN header (draft-smith-vxlan-group-policy). It adds a Group Policy ID (SGT) to the VXLAN header for micro-segmentation.

Standard VXLAN (RFC 7348): flags byte = 0x08 (I-bit only)

VXLAN-GBP (Cisco SGT / Versa): flags byte = 0x88 (G-bit + I-bit)

| Bit | Name | Description |
| --- | --- | --- |
| G (bit 0) | GBP Extension | When set, indicates GBP header present |
| D (bit 3) | Don't Learn | Tells remote VTEP not to learn source MAC |
| I (bit 4) | VNI Valid | Same as standard VXLAN |
| A (bit 7) | Applied | Policy already applied at source |
| Group Policy ID | SGT | 16-bit Security Group Tag |

### 11.2 Packet Capture Evidence

#### Evidence A: Versa FlexVNF -> Cumulus (GBP always on)

Captured on sw1 spine between Cumulus (1.1.1.5) and Versa (1.1.1.4):

Frame  Source (outer)           Destination (outer)      Flags   VNI

-----  ----------------------  -----------------------  ------  ------

11     1.1.1.5 Cumulus      -> 1.1.1.4 Versa            0x0800  401151

12     1.1.1.4 Versa        -> 1.1.1.5 Cumulus           0x8808  401151

13     1.1.1.5 Cumulus      -> 1.1.1.4 Versa            0x0800  401151

14     1.1.1.4 Versa        -> 1.1.1.5 Cumulus           0x8808  401151

...    (pattern repeats for ALL packets)

#### Evidence B: Cisco sw2 with SGT -> Cumulus (GBP + SGT 100)

Captured on Cumulus swp1 with tcpdump -i swp1 -n 'udp port 4789' -XX -vvv:

# Cisco sw2 (1.1.1.2, group-based-policy ON) -> Cumulus (1.1.1.5)

# ICMP Echo Reply: 192.168.1.22 -> 192.168.1.11

VXLAN Header (hex): 88 00 00 64 06 1e ff 00

^^       ^^^^^

|        Group Policy ID = 0x0064 = SGT 100

Flags = 0x88 (G-bit + I-bit = GBP ENABLED)

# Cisco sw3 (1.1.1.3, NO group-based-policy) -> Cumulus (1.1.1.5)

# ICMP Echo Reply: 192.168.1.23 -> 192.168.1.11

VXLAN Header (hex): 08 00 00 00 06 1e ff 00

^^

Flags = 0x08 (I-bit only = STANDARD VXLAN)

Key Finding: Cisco sw2 with group-based-policy + CTS/SGT sends identical GBP flags (0x88) as Versa FlexVNF. The only difference is the Group Policy ID value (Cisco inserts the configured SGT tag, e.g., 100). Both cause the same packet drops on Cumulus Linux without the gbp flag.

### 11.3 Raw Hex Dump Comparison

Flags Rsvd  GPID    VNI      Rsvd

----- ----  ------  -------  ----

Cisco sw2 (GBP):   88    00    00 64   06 1e ff 00    <- SGT 100 in GPID field

Cisco sw3 (std):   08    00    00 00   06 1e ff 00    <- No GBP, GPID=0

Cumulus (std):     08    00    00 00   06 1e ff 00    <- No GBP, GPID=0

Versa (GBP):      88    00    xx xx   06 1e ff 00    <- GBP enabled, GPID varies

### 11.4 Why This Causes Failure

The Linux kernel VXLAN driver validates header flags strictly. Without the gbp option, the kernel expects flags = 0x08 (I-bit only). Receiving 0x88 (G-bit set) causes the packet to be silently dropped. No error, no log, no counter increment, no ICMP unreachable. This affects all GBP-sending VTEPs equally: Cisco with SGT, Versa, or any other vendor using VXLAN-GBP.

GBP-Sending VTEP       Remote Client

Cumulus Client   Cumulus VTEP               (Cisco SGT / Versa)    192.168.1.x

192.168.1.12     1.1.1.5                    1.1.1.2 or 1.1.1.4

|               |                          |                      |

|-- ICMP Req -> |                          |                      |

|               |== VXLAN flags=0x08 =====>|                      |

|               |     (standard)           |-- ICMP Req -------> |

|               |                          |                      |

|               |                          |<-- ICMP Reply ------ |

|               |<= VXLAN flags=0x88 ======|                      |

|               |     (GBP!)               |                      |

|            [​DROPPED]                     |                      |

|         kernel rejects GBP               |                      |

|         on non-GBP iface                 |                      |

### 11.5 Affected Traffic Flows

| Sender | Receiver | VXLAN Flags | Result |
| --- | --- | --- | --- |
| [CISCO] sw2 (GBP+SGT ON) | [CUMULUS] | 0x88 | DROPPED (without gbp flag) |
| [VERSA] FlexVNF | [CUMULUS] | 0x88 | DROPPED (without gbp flag) |
| [CISCO] sw3 (GBP OFF) | [CUMULUS] | 0x08 | OK |
| [CUMULUS] | Any VTEP | 0x08 | OK |
| [CISCO] sw2 (GBP+SGT ON) | [CISCO] sw3 | 0x88 | OK (IOS-XE ignores GBP) |

Information: Cisco IOS-XE Receivers: Not Affected. IOS-XE silently ignores GBP bits and processes packets normally. This issue only affects Linux kernel VXLAN (Cumulus, Ubuntu, etc.).

Cisco as a Sender: Same Impact as Versa: When group-based-policy is enabled under interface nve1 on Cisco Catalyst 9300, it sends VXLAN-GBP headers (flags=0x88) with SGT tag. This causes the exact same packet drops on Cumulus as Versa does. The issue is symmetric — any GBP sender triggers the Linux kernel drop.

## 12. VXLAN-GBP Fix & Resolution

| Attribute | Detail |
| --- | --- |
| Affected Traffic | Any GBP-sending VTEP -> Cumulus Linux (data plane only) |
| GBP Senders | Cisco Catalyst 9300 (with group-based-policy + CTS/SGT), Versa FlexVNF (GBP always on) |
| Root Cause | Linux kernel VXLAN driver drops packets with G-bit set (flags=0x88) on non-GBP interfaces. Not vendor-specific. |
| Control Plane | Not affected — BGP EVPN routes exchanged correctly |
| Cisco as Receiver | Not affected — IOS-XE ignores GBP bits from other senders |
| Cisco as Sender | Causes same drop on Cumulus when SGT/GBP enabled (flags=0x88, SGT in GPID field) |
| Fix Required On | Cumulus Linux (any Linux-based VTEP without gbp flag) |

### 12.1 Step 1 — Recreate vxlan48 with GBP

# Delete existing

sudo ip link del vxlan48

# Recreate with GBP flag

sudo ip link add vxlan48 type vxlan id 401151 local 1.1.1.5 dstport 4789 nolearning gbp

sudo ip link set vxlan48 up

### 12.2 Step 2 — Restore bridge and VLAN settings

# Add back to bridge

sudo ip link set vxlan48 master br_default

# CRITICAL: Fix VLAN mapping (default PVID=1, must be 1151)

sudo bridge vlan del vid 1 dev vxlan48

sudo bridge vlan add vid 1151 dev vxlan48 pvid untagged

# Restore bridge port settings

sudo bridge link set dev vxlan48 state 3          # forwarding

sudo ip link set vxlan48 mtu 9216                 # match fabric MTU

sudo bridge link set dev vxlan48 learning off      # EVPN controls FDB

sudo bridge link set dev vxlan48 neigh_suppress on # ARP suppression

sudo bridge link set dev vxlan48 vlan_tunnel on

Alert: VLAN Mapping Pitfall: When vxlan48 is recreated, the bridge assigns VLAN 1 as default PVID. This maps VNI 401151 to VLAN 1 instead of 1151, breaking ALL VXLAN traffic. Must explicitly fix.

Warning: Persistence: The manual ip link recreation is not persistent across reboots. NVUE does not currently expose a gbp option. Configure a post-boot script or edit /etc/network/interfaces.

### 12.3 Post-Fix Verification

cumulus@cumulus:~$ ip -d link show vxlan48 | grep -i gbp

vxlan id 401151 local 1.1.1.5 dstport 4789 nolearning ttl 64 udpcsum gbp

cumulus@cumulus:~$ bridge vlan show dev vxlan48

port     vlan ids

vxlan48  1151 PVID Egress Untagged

cumulus@cumulus:~$ sudo vtysh -c "show evpn vni 401151" | grep Vlan

Vlan: 1151          <- must be 1151, NOT 1

Status: Resolution Confirmed: After enabling GBP, Cumulus accepts VXLAN-GBP packets from all GBP-sending VTEPs (Cisco with SGT and Versa). Full connectivity across all VTEPs.

## 13. Command Reference

### Cisco IOS-XE

| Command | Purpose |
| --- | --- |
| show bgp l2vpn evpn summary | BGP EVPN neighbor status |
| show bgp l2vpn evpn route-type 2 | MAC/IP routes (Type-2) |
| show bgp l2vpn evpn route-type 3 | IMET routes (Type-3) |
| show nve peers | VXLAN tunnel peer status |
| show nve vni | VNI status / mappings |
| show l2vpn evpn mac evi <id> | EVPN learned MACs per EVI |
| show l2vpn evpn evi detail | Detailed EVI information |
| show ip ospf neighbor | OSPF adjacency status |

### Cumulus Linux (NVIDIA)

| Command | Purpose |
| --- | --- |
| sudo vtysh -c "show bgp l2vpn evpn summary" | BGP EVPN peers |
| sudo vtysh -c "show evpn vni <id>" | VNI status (VLAN map, remote VTEPs) |
| sudo vtysh -c "show evpn mac vni <id>" | EVPN MAC table per VNI |
| bridge fdb show dev vxlan48 | Bridge FDB (MAC table) |
| bridge vlan show dev vxlan48 | VLAN membership check |
| ip -d link show vxlan48 | VXLAN interface detail (check GBP) |
| nv config show --output commands | Running config in set format |

### Versa FlexVNF

| Command | Purpose |
| --- | --- |
| show bgp summary | BGP neighbor status |
| show bridge | MAC address table |
| show bridge-evpn | EVPN instance status |
| show ospf route routing-instance Underlay | OSPF routes |

## 14. Multi-Vendor Configuration Comparison

This section provides side-by-side comparison of configurations and verification outputs across all three vendor platforms for the same EVPN VXLAN fabric functions.

### 14.1 OSPF Underlay Configuration

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| router ospf 100 router-id 1.1.1.2 auto-cost reference-bandwidth 100000 interface Loopback0 ip address 1.1.1.2 255.255.255.255 ip ospf 100 area 100 interface GigabitEthernet1/0/1 description to sw1 no switchport ip address 12.12.12.2 255.255.255.252 ip ospf network point-to-point ip ospf 100 area 100 | set routing-instances Underlay instance-type virtual-router networks [​ OSPF_Underlay ] interfaces [​ tvi-0/9005.0 ] evpn-core set routing-instances Underlay protocols ospf 4023 router-id 12.12.12.10 area 100 network-name OSPF_Underlay network-type point-to-point hello-interval 10 dead-interval 40 set routing-instances Underlay policy-options redistribution-policy Export-Vtep-To-OSPF term VTEP_IP match address 1.1.1.4/32 action accept redistribute-to-ospf Export-Vtep-To-OSPF | # NVUE Commands: nv set router ospf enable on nv set router ospf router-id 1.1.1.5 nv set vrf default router ospf enable on nv set vrf default router ospf router-id 1.1.1.5 nv set interface lo router ospf enable on nv set interface lo router ospf area 100 nv set interface swp1 router ospf enable on nv set interface swp1 router ospf area 100 nv set interface swp1 router ospf network-type point-to-point nv set interface swp1 router ospf mtu-ignore on # FRR Result: router ospf ospf router-id 1.1.1.5 interface swp1 ip ospf area 100 ip ospf mtu-ignore ip ospf network point-to-point |

### 14.2 BGP EVPN Overlay Configuration

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| router bgp 6500 bgp router-id interface Loopback0 bgp log-neighbor-changes neighbor 1.1.1.1 remote-as 6500 neighbor 1.1.1.1 update-source Loopback0 address-family l2vpn evpn neighbor 1.1.1.1 activate neighbor 1.1.1.1 send-community both exit-address-family | set routing-instances Underlay protocols bgp 3023 router-id 1.1.1.4 local-as as-number 64515 group vxlan_internal type internal family l2vpn evpn local-as 6500 neighbor 1.1.1.1 local-address tvi-0/9005.0 | # NVUE Commands: nv set router bgp enable on nv set router bgp autonomous-system 6500 nv set router bgp router-id 1.1.1.5 nv set vrf default router bgp enable on nv set vrf default router bgp neighbor 1.1.1.1 remote-as internal nv set vrf default router bgp neighbor 1.1.1.1 update-source lo nv set vrf default router bgp address-family l2vpn-evpn enable on nv set vrf default router bgp neighbor 1.1.1.1 address-family l2vpn-evpn enable on # FRR Result: router bgp 6500 bgp router-id 1.1.1.5 neighbor 1.1.1.1 remote-as internal neighbor 1.1.1.1 update-source lo address-family l2vpn evpn neighbor 1.1.1.1 activate advertise-all-vni |

### 14.3 VXLAN / L2VPN Instance Configuration

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| l2vpn evpn instance 1151 vlan-based encapsulation vxlan route-target export 1151:1 route-target import 1151:1 route-target import 2:2 vlan configuration 1151 member evpn-instance 1151 vni 401151 interface nve1 no ip address source-interface Loopback0 host-reachability protocol bgp member vni 401151 ingress-replication | set routing-instances VERSA-default-switch instance-type virtual-switch bridge-options l2vpn-service-type vlan bridge-domains VLAN-1151 vlan-id 1151 bridge-options bridge-domain-arp-suppression enable vxlan vni 401151 interfaces [​ vni-0/1.1 ] route-distinguisher 2L:122 vrf-both-target "target:2L:2 target:1151:1" protocols evpn vlan-id-list [​ 1151 ] encapsulation vxlan core-instance Underlay | # NVUE Commands: nv set nve vxlan enable on nv set nve vxlan source address 1.1.1.5 nv set evpn enable on nv set bridge domain br_default vlan 1151 vni 401151 nv set evpn vni 401151 route-target export 1151:1 nv set evpn vni 401151 route-target import 1151:1 # /etc/network/interfaces Result: auto vxlan48 iface vxlan48 bridge-vlan-vni-map 1151=401151 bridge-learning off auto br_default iface br_default bridge-ports swp2 vxlan48 bridge-vlan-aware yes bridge-vids 1151 |

### 14.4 OSPF Neighbor Verification

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| sw2# show ip ospf neighbor Neighbor ID Pri State Dead Time Address Interface 1.1.1.1 0 FULL/ - 00:00:35 12.12.12.1 Gi1/0/1 | admin@Cisco-EVPN-Br1> show ospf Router ID Version Admin Op status Instance 12.12.12.10 2 en up 4023 Intf address Interface State Neighbor ID Pri Op 12.12.12.9 vni-0/2.0 full 1.1.1.1 1 up | cumulus$ sudo vtysh -c "show ip ospf neighbor" Neighbor ID Pri State Dead Time Address Interface 1.1.1.1 1 Full/DROther 35.993s 12.12.12.13 swp1:12.12.12.14 |

### 14.5 BGP EVPN Peer Verification

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| sw2# show bgp l2vpn evpn summary Neighbor V AS MsgRcvd MsgSent TblVer InQ OutQ Up/Down State 1.1.1.1 4 6500 844 816 225 0 0 11:59:50 14 1.1.1.3 4 6500 822 822 225 0 0 11:59:47 17 | admin@Cisco-EVPN-Br1> show bgp summary routing-instance: Underlay BGP instance 3023 router-id : 1.1.1.4 l2vpn evpn statistics: iBGP routes in : 25 Active routes : 23 Advertised routes : 1 Neighbor State AS Up/Down 1.1.1.1 estab 6500 12:00:01 | cumulus$ sudo vtysh -c "show bgp l2vpn evpn summary" BGP router identifier 1.1.1.5 local AS number 6500 vrf-id 0 Peers 1, using 23 KiB of memory Neighbor V AS MsgRcvd MsgSent Up/Down State/PfxRcd PfxSnt 1.1.1.1 4 6500 13501 13291 10:01:22 25 2 |

### 14.6 VXLAN Tunnel / NVE Peer Verification

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| sw2# show nve peers Interface VNI Type Peer-IP eVNI state UP time nve1 401151 L2CP 1.1.1.3 401151 UP 00:43:10 nve1 401151 L2CP 1.1.1.4 401151 UP 00:40:58 nve1 401151 L2CP 1.1.1.5 401151 UP 00:43:10 | admin@Cisco-EVPN-Br1> show bridge ingress-table routing-instance VERSA-default-switch BDOMAIN INTERFACE ENCAP OR VNI TUNNEL VLAN-1151 vni-0/1.1 N/A N/A N/A dtvi-0/79 VXLAN 401151 1.1.1.2 dtvi-0/80 VXLAN 401151 1.1.1.3 dtvi-0/81 VXLAN 401151 1.1.1.5 | cumulus$ sudo vtysh -c "show evpn vni 401151" VNI: 401151 Type: L2 Vlan: 1151 VxLAN interface: vxlan48 Local VTEP IP: 1.1.1.5 Remote VTEPs for this VNI: 1.1.1.4 flood: HER 1.1.1.3 flood: HER 1.1.1.2 flood: HER |

### 14.7 MAC Address Table Verification (VLAN 1151)

| Cisco Catalyst 9300 (sw2) | Versa FlexVNF | Cumulus Linux (NVIDIA) |
| --- | --- | --- |
| sw2# show l2vpn evpn mac evi 1151 MAC Address EVI VLAN Next Hop(s) 5254.0091.2c07 1151 1151 1.1.1.3 5254.00c4.b4a1 1151 1151 Gi1/0/3:1151 aabb.cc00.2400 1151 1151 Gi1/0/4:1151 aabb.cc00.2700 1151 1151 1.1.1.3 aabb.cc00.3010 1151 1151 1.1.1.5 | admin@Cisco-EVPN-Br1> show bridge mac-table brief ROUTING INSTANCE BRIDGE INTERFACE MAC-ADDRESS TYPE VLAN TUNNEL EP VERSA-default-switch VLAN-1151 dtvi-0/79 aa:bb:cc:00:24:00 CA 1151 1.1.1.2 VERSA-default-switch VLAN-1151 dtvi-0/80 aa:bb:cc:00:27:00 CA 1151 1.1.1.3 VERSA-default-switch VLAN-1151 dtvi-0/81 aa:bb:cc:00:30:10 CA 1151 1.1.1.5 VERSA-default-switch VLAN-1151 vni-0/1.1 aa:bb:cc:00:2f:10 DA 1151 N/A | cumulus$ sudo vtysh -c "show evpn mac vni 401151" MAC Type Intf/Remote aa:bb:cc:00:27:00 remote 1.1.1.3 aa:bb:cc:00:30:10 local vlan1151 aa:bb:cc:00:24:00 remote 1.1.1.2 aa:bb:cc:00:2f:10 remote 1.1.1.4 52:54:00:c4:b4:a1 remote 1.1.1.2 52:54:00:91:2c:07 remote 1.1.1.3 |

## 15. Lessons Learned

| # | Issue | Impact | Resolution |
| --- | --- | --- | --- |
| 1 | VXLAN-GBP Flag Mismatch | 100% data plane failure from any GBP-sending VTEP (Cisco with SGT, Versa) to Cumulus. Linux kernel VXLAN driver behavior, not vendor-specific. | Enable gbp on Cumulus VXLAN interface at creation time |
| 2 | VLAN-VNI Mapping after Recreation | VNI maps to VLAN 1 instead of 1151 | bridge vlan del vid 1, then add vid 1151 pvid untagged |
| 3 | OSPF MTU Mismatch | OSPF stuck in ExStart (Cisco 1550 vs Cumulus 9216) | ip ospf mtu-ignore on both ends |
| 4 | Cumulus NVUE OSPF Context | OSPF daemon won't activate | Must use: nv set vrf default router ospf enable on |
| 5 | Versa Route Target | Versa exports RT 2:2 + 1151:1 | All VTEPs import 1151:1. Optionally import 2:2. |
| 6 | swp2 Access vs Trunk Mode | Client uses tagged VLAN 1151. Access port sends replies untagged, client drops them. | Keep swp2 as trunk: VLAN 1 native PVID + VLAN 1151 tagged. Do NOT use access mode. |

### Recommendations

#### For Linux-based VTEPs (Cumulus, Ubuntu, etc.)

Always create VXLAN interfaces with gbp flag when integrating with any GBP-capable VTEP (Cisco with SGT, Versa, or others).

Without gbp, the Linux kernel silently drops GBP-flagged packets — no logs, no counters, extremely difficult to diagnose.

GBP Persistence: Edit /etc/network/interfaces or create post-boot script. NVUE does not support gbp natively.

Monitoring: Check VXLAN flags after every nv config apply to ensure GBP preserved.

#### For Cisco Catalyst 9300 with SGT/GBP

Enabling group-based-policy on NVE changes the VXLAN header format from standard (0x08) to GBP-extended (0x88).

This will break connectivity to any Linux-based VTEP that lacks the gbp interface flag.

If SGT microsegmentation is not required, leave group-based-policy disabled to maintain standard VXLAN interoperability.

#### For Multi-Vendor Fabrics

Test data plane connectivity after any GBP/SGT configuration change — control plane (BGP EVPN) will appear healthy even when data plane is completely broken.

Packet Captures: Use tcpdump -i <intf> -n 'udp port 4789' -XX to verify VXLAN flags byte (0x08 = standard, 0x88 = GBP).

The issue is symmetric — Cisco with SGT causes the same drop on Cumulus as Versa does.

MTU: Standardize underlay MTU across all vendors.

INTERNAL ENGINEERING DOCUMENT

Multi-Vendor EVPN VXLAN — v3.0 — February 2026
Generated with Claude Code